CTMAP 0.99d - DOS Memory manager/extender for 80286, 80386, 80386SX and

80486 "AT" computers which use Chips & Technologies DRAM

controller chips.

CTLOADHI 0.99c - Makes memory-resident software load in "Upper" RAM.

Especially useful for network drivers like Novell's IPX,

NET3 and NET4.

Copyright 1989-1992, Burton Systems Software.  All rights reserved.

Burton Systems Software

P.O. Box 4156, Cary, NC 27519 USA

Tel: (919) 233-8128    FAX: 233-0716    BBS: 233-0106

*** Note: Version 0.99d supports several additional C&T DRAM controllers.

OVERVIEW

--------

The CTMAP package contains two programs: CTMAP and CTLOADHI.

CTMAP:

If you are using an 80286, 80386, 80386SX or 80486 computer which has a

Chips & Technologies 82C212 or 82C812 "NEAT" chip set, an 82C235 or 82C836

"SCAT" chip, an 82C302 or 82C307 "CHIPSet", or an 82C351 "PEAK DM" DRAM

controller chip, then CTMAP can (probably) increase the amount of usable

RAM memory available to DOS from the usual 640K to as much as 944K,

depending on what else you have installed in your computer.

If you do not use EGA or VGA graphics, then 64 or 96K of this additional

memory will usually be contiguous with the 640K of "low" memory which DOS

normally uses, increasing that area to 704K or 736K.  Additionally, CTMAP

can enable DOS to use up to 240K of discontiguous RAM, located above the

display adapter RAM and below the ROM BIOS.  However, some programs can't

use discontiguous RAM (and CHKDSK ignores it).

CTMAP has a significant advantage over other commercial 80386-only memory

managers like 386-MAX and QEMM (which are trademarks or their respective

vendors).  CTMAP does not put the CPU into protected mode, nor does it

install a TSR or device driver, so it is HIGHLY COMPATIBLE with protected-

mode 80286/80386/80486 software.

In fact, it even works in the OS/2 1.2 and 1.3 "DOS Compatibility Box."

CTLOADHI:

CTLOADHI is used to make programs (especially TSRs) load in "upper" memory

(the RAM that CTMAP enables above 640K, between the video display RAM and

the ROM BIOS).

By installing your TSRs in upper memory, you can further increase the

amount of contiguous low memory available for those application programs

which will not directly utilize upper memory.

Note: CTLOADHI used to be called LOADHIGH, but we renamed it because there's

an internal command by the same name in MS-DOS 5.0.

CTMAP

-----

For operating instructions, run CTMAP like this:

CTMAP /?

Typical usage with EGA or VGA adapter is:

CTMAP /A -A -B

Typical usage with non-VGA monochrome adapter is:

CTMAP /A

If you use "EMS"/"XMS"/"LIM"/"expanded" memory on your computer, you may

also have to add either the "-D" switch or the "-E" switch prevent CTMAP

from interfering with it.

CTLOADHI

--------

For operating instructions, run CTLOADHI like this:

CTLOADHI /?

TERMINOLOGY

-----------

These seem to be the most generally accepted terms for the various kinds of

memory on IBM-compatible PCs:

K or kilobyte   = 2**10 = 1024 bytes.  But some people use this term to mean

10**2 = 1000 bytes.  In this document, we use the regular

"1024" meaning.  It is a curious fact that 2**10 & 10**2

are almost (but not quite) equal.

M or megabyte   = 1024K = 1024x1024 = 1,048,576 bytes.  But occasionally this

term is used to mean 1,000,000 bytes (which is why your

"100 megabyte" disk drive may only hold 95.4 megabytes).

EXTENDED MEMORY = RAM which is above 1M+64K.  But sometimes this term is used

to refer to all memory above 1M, including the HIGH memory.

Not available on 8088 ("XT") computers.

HIGH MEMORY     = 64K of RAM above 1M and below 1M+64K (used by Microsoft

HIMEM.SYS driver).  However, this term is also sometimes

used to refer to UPPER memory (below).  Not available on

8088 computers.

UPPER MEMORY    = RAM which is above the DISPLAY memory but below the BIOS.

Usually between 756K and 960K.  Sometimes called "high

memory."  Earlier versions of CTMAP used the obsolete term

"high memory" to refer to this region.  Now we call it

"upper memory," but we still say that if a program ran

in upper memory, it "loaded high."

EXPANDED MEMORY = Any of several types of additional RAM which is accessed

via a "page frame segment" (usually 64K long) in the UPPER

memory region.  Also called EMS, XMS, or LIM memory.

DISPLAY MEMORY  = The special RAM used by display adapters, generally

addressed somewhere between 640K and 764K.

LOW MEMORY      = The normal DOS RAM between 0K and the DISPLAY memory.

HOW IT WORKS

------------

CTMAP is a simple program.  Mainly, what it does is manipulate the

hardware registers in the C&T DRAM controller chip to enable the "extra"

RAM which is already there on your computer, then it patches up the DOS

memory control block chain so that DOS "knows about it."

The DOS memory control block chain is documented in a number of places.

See, for instance, the Microsoft's MS-DOS Encyclopedia, the Turbo Pascal

source code for Kim Kokkonen's famous MAPMEM utility, or the excellent

book "Undocumented DOS," by Schulman, Kyle, et al (ISBN 0-201-57064-5,

Addison-Wesley).

CTLOADHI is even simpler than CTMAP.  Mainly what it does is change the

"owner" fields in DOS memory control blocks so that DOS considers them

used or available.

CAVEATS, PROBLEMS, ETC.

-----------------------

1) No warranties of any kind are expressed or implied.  Use at your own risk.

2) If you find that some I/O device or an EMS/EMM/LIM "expanded" memory

card or driver fails when you use "CTMAP /A", you can use explicit "-xx"

options to prevent CTMAP from using particular areas of RAM.  We suggest

trial and error to determine which memory segments you can and cannot

allow CTMAP to use.  For display adapters, the most likely culprits are

the A and B segments.  For EMS/EMM/LIM (expanded memory) problems, the D

or E segment (but not both!) is usually at fault.  Examples:

VGA/EGA systems:        CTMAP -A -B /A

For EMM at D000:        CTMAP -D /A

For EMM at E000:        CTMAP -E /A

VGA/EGA + EMM at D000:  CTMAP -A -B -D /A

VGA/EGA + EMM at E000:  CTMAP -A -B -E /A

For how to use CTMAP's -xx options, run "CTMAP /?".

3) A few TSRs may not work when loaded in upper memory.  In particular,

TSRs which increase the size of the keyboard type-ahead buffer probably

will not work when loaded in upper memory.

4) We've noticed that some command line editor programs get confused

about which retrieval buffer to use if they are loaded in upper memory or

if they are loaded before CTMAP and CTLOADHI are run.  DOSEDIT, NDOSEDIT

and RETRIEVE have this problem, so if you use one of them, you should

install it at the end of your AUTOEXEC.BAT, after the "CTLOADHI OFF".

However, CED and PCED (two other popular command line editors) do not

seem to have this problem.  We've not yet tried ANARKEY or ALIAS (other

command line editors).

5) You can use CTMAP/CTLOADHI to load DEVICE DRIVERS into upper memory,

but you'll need to also use another program: DEVLOD, by Jim Kyle.  DEVLOD

allows you to load device drivers from the DOS command line (e.g., from

your AUTOEXEC.BAT file), instead of via your CONFIG.SYS file.  By running

CTLOADHI and then using DEVLOD to load your device drivers, you can make

them load into upper RAM.

DEVLOD.COM is included in this package by permission of the copyright

owners, Jim Kyle and Addison-Wesley Publishing Company, Inc.  This

program first appeared in "Undocumented DOS", ISBN 0-201-57064-5, and

the book includes full source code for it together with many other

useful programs and information.  To order a copy or obtain additional

information, contact the publisher at 1-800-447-2226.

See DEVLOD.DOC for details.

6) We're using undocumented DOS features to manipulate the DOS memory

pool, so we cannot guarantee compatibility with future DOS releases.

If you have a problem, let us know - perhaps we can solve it.

7) We recommend the use of Chris Dunford's PMAP program to display the

DOS memory pool.  It is very similar to Kim Kokkonen's MAPMEM, but it

does a better job of displaying information about "upper" memory.  If

you received CTMAP on diskette, you may have also received a copy of

PMAP; otherwise, look for PMAP132.ZIP on your favorite BBS.

However, PMAP is not a part of the CTMAP package!  PMAP is a different

product, from a different company; see PMAP.DOC for license information

and legal restrictions on the use and distribution of PMAP.

8) Note that some programs, such as MAPMEM and DOS CHKDSK, will not

display information about DOS RAM above the video display.  Also,

Norton-SI will tell you about upper RAM... but will not tell you whether

it is part of the DOS memory pool.  This does not mean that CTMAP did

not work!  You can run "CTLOADHI OFF" for a quick display of available

RAM (do it before and after running CTMAP to see the difference!).

Better yet, use Chris Dunford's PMAP (for details, use "PMAP /R").

LANs

----

Most local area network adapters use some upper address space.  CTMAP

will generally detect the LAN adapter RAM and/or ROM, and not enable

general purpose RAM at that address.  In other words, CTMAP will

probably not interfere with your LAN, but the presence of the LAN

adapter will probably reduce the amount of RAM which CTMAP can add to

DOS's memory pool.

If you have trouble runing your Local Area Network when CTMAP is active,

it is probably because CTMAP has enabled conventional RAM in a region of

memory which the network adapter uses for its buffer.  Normally, CTMAP

would detect the presence of the network adapter's buffer, and would

exclude that region from the added DOS RAM, thus avoiding the problem.

However, some network adapters "hide" their buffers when the network is

not running, which causes CTMAP to use the buffer's address region for

conventional RAM, which interferes with the network.

The easiest way to determine whether your adapter does this is to run

CTMAP twice with the /? (help) option: run it once before starting the

network, and once after.  On the sixth help screen, CTMAP will tell you

"The additional high RAM which appears to be available on your computer

is:...".  If tells you that less high RAM is available when the network

is running, then you'll know that your network adapter hides its buffer

when not in use, and you'll know which addresses need to be excluded

via CTMAP's "-xx" options to avoid interferring with the network.

If you can choose the memory address for your LAN card, it is a good

idea to try to pick an address adjacent to either the ROM BIOS (at

F0000) or the display RAM (the address depends upon your display

adapter).  This will allow CTMAP to add the upper memory to DOS's

memory pool as one big chunk of memory instead of two little chunks,

so that bigger programs will fit in it, with less wasted RAM.

Example:

We once used Novell NetWare on an Arcnet, using Network Innovations'

PcARC network adapter.  It uses 16K of RAM at an address which is

selected by DIP switches on the board.  The most common setting, and

the one expected by Novell's ANET3.COM program is:

D0000 (the default)

However, we relocated it to:

EC000 (just below the BIOS RAM)

(Note: the Network Innovations documentation refers to these addresses

as D000H and EC00H.)

To use the reconfigured card with one version of Novell NetWare, it was

necessary to patch ANET3.COM.  That version of NetWare uses a 50605 byte

version of ANET3.COM, dated 05/16/88.  When run with the "I" option it

displays:

Advanced/SFT NetWare V2.01-2 Workstation Shell for PC DOS V3.x

Copyright (c) by Novell, Inc.  1983, 1987

LAN Option: Standard Microsystems ARCNET

That version of ANET3.COM is fairly easy to patch (if you are comfortable

using DOS debug).  Simply use DOS debug to search the file for the eight

instances of 00 D0, and change all eight instances to 00 EC.

Later, we upgraded to a newer version of NetWare which had to be patched

differently.  Instead of ANET3.COM, it has programs named IPX.COM and

NET3.COM (or NET4.COM).

IPX.COM is the program which needs to be patched.  Ours was 27966 bytes

long, dated 8/25/89.  When installed for our LAN card, it displayed:

Novell IPX/SPX V2.15

(C) Copyright 1985, 1988 Novell Inc.  All Rights Reserved.

LAN Option: Standard Microsystems ARCNET/Pure Data  V1.00

Hardware Configuration: IRQ = 2, I/O Base = 2E0h, RAM Buffer at D000:0

In our case, it was easy to patch the IPX.COM memory address.  We simply

used DOS debug to search the file for the first instance of 00 D0, and

we changed it to the desired address (00 EC, in our case).  For esthetics,

we also patched the string which follows to indicate the proper address

("EC00:0" instead of "D000:0"), so that the patched version of IPX.COM

displays the proper address when it is installed:

Hardware Configuration: IRQ = 2, I/O Base = 2E0h, RAM Buffer at EC00:0

IPX.COM contains a table of 18 combinations of hardware settings; the spot

we patched was the first entry in the table.  If your LAN uses a different

interrupt level (IRQ) or I/O Base, you'll have to patch a different table

entry.

LICENSE

-------

This software is not free.  You may try it out on your computer(s) at no

cost, but if you like it and decide to continue using it, or if you add

it to your AUTOEXEC.BAT, then you must register it at a cost of $30 per

computer.

REGISTRATION

------------

Register by sending $30 per computer to:

CTMAP 0.99d registration

Burton Systems Software

P.O.Box 4156, Cary, NC 27519  USA

Please include the following:

Your full name, your company (if any), your telephone number(s), your

mailing address (including country), the number of computers you are

registering for use of CTMAP 0.99d, the brands and models of your

computers & your version of DOS, and your payment of $30 US (or $40

Canadian) per computer.  Also, please answer the questions:  "Are you

a programmer?  In what computer languages?"

Payment should be by check or money order on a US or Canadian bank, or

by Visa, MasterCard or EuroCard charge.  If paying by credit card, please

be sure to include your credit card number, expiration date, type of card

(Visa or MC), and the name on the card; and please sign your letter.  Or,

with a credit card, you can register by telephone or FAX.  Our telephone

is (919) 233-8128, our FAX is (919) 233-0716, our BBS is (919) 233-0106.

DISTRIBUTION

------------

You are encouraged to give copies of this software to your friends and

colleagues for trial on their computers.  You are free to "upload" this

software to any computer bulletin board system (BBS).  You may charge

recipients of this software a fee of no more than $6 US plus the cost

of the distribution media, provided that purchasers are forewarned that

continued use requires registration with Burton, and that registration

costs $30. However, under no circumstances may "patched" or otherwise

modified copies of this software be distributed without prior written

permission from Burton.

Manufacturers and distributors of computer equipment should contact

Burton to purchase distribution licenses for customized versions of this

software.

DISCLAIMER OF WARRANTY

----------------------

No warranty is expressed or implied.  Use this software at your own risk.

However, if you encounter problems, or if you have suggestions for

improvement, we would like to hear from you.

WHO ARE WE?

-----------

Burton Systems Software is a commercial software development company

located in Raleigh, North Carolina.  We specialize in systems software

and software development tools for programmers.  We are best known for our

TLIB (tm) Version Control System for MS-DOS and OS/2.  TLIB is the fastest,

most powerful, and easiest to use version control software available for

PCs.  It elegantly solves many of the headaches attendant to software

development.  But don't take our word for it - give us a call, and we'll

gladly send you more information, including a long list of quotes from

enthusiastic magazine reviewers.  Or try it out - TLIB comes with a full

90 day warranty (unlike CTMAP!).  Our telephone number is (919) 233-8128.

BONUS!

------

When you register CTMAP, you'll receive a coupon worth $30 off the price

of a TLIB Version Control System.  You may also receive an updated

version of CTMAP (at our option), if we have improved it significantly

since February, 1992.

Note to TLIB customers: if you got CTMAP with TLIB, you can register for

free - sort of a pre-redeemed coupon.  Just mention your TLIB serial

number in a letter which says "I'm using CTMAP; please register me."